| Command String | Parameter Name | Example Value | Valid Range | Default values | Description |
| --- | --- | --- | --- | --- | --- |
| @R_50# | Speed Limit | 50 | 0 .. 199 | 40 km/hr | Sets the maximum speed limit (e.g., vehicles exceeding 50 km/h will be flagged). |
| @H_10# | Speed Threshold | 10 | 0 .. 199 | 1km/hr | Minimum speed threshold; ignores objects moving slower than 10 km/h. |
| @A_15# | Installation Angle | 15 | 0 .. 45 | 0 deg | Radar mounting/installation angle in degrees. |
| @D_30# | Sensitivity | 5 | 1 .. 15 | 5 | Maximum Sensitivity range in meters. |
| @N_01# | Detection Direction | 1 | 0 .. 2 | 1 | 0=receding,1=approaching,2=both) |
| @F_1# | Sampling Frequency | 1 | 0 .. 1 | 0 (22sample/sec) | Radar sampling frequency (Hz). |
| @U_01# | Speed Unit | 1 | 0 .. 1 | 0 | Sets speed display unit: 0 = km/h, 1 = mph. |

BAUDRATE:- 115200bps

Typical Responses on Terminal

When you send a setting command (for example @H_10#), the radar and controller reply with short messages on the serial terminal.
Here’s how to read them:

| Response Seen on Terminal | Meaning for User |
| --- | --- |
| Setting saved successfully! | The radar accepted your command and the new setting is active. |
| Setting NOT saved, retrying... | The radar received your command but didn’t apply the setting. Try sending it again. |
| Failed to save setting after retries. | The radar did not accept the setting after multiple attempts. |
| Invalid frame received! | The reply was incomplete or corrupted. Try again. |
| No response / Timeout | The radar did not reply. Check power, wiring, and serial connection. |

Quick Debug Checklist

- Send Command

  - Type the setting command in the serial terminal.
Example:

    - @R_50# → sets speed limit to 50

    - @A_15# → sets installation angle to 15°

- Check the Reply

  - If you see “Setting saved successfully!” → ✅ The setting was applied.

  - If you see “Setting NOT saved or mismatch” → resend the command.

  - If you see “Invalid frame” → resend once or restart the radar.

  - If you see  “Failed to save setting after retries” or ”no response → check wiring, serial port, or power.

- If Problems Continue

  - Make sure the value you entered is within the valid range shown in the manual.

  - Reconnect the cables and restart both radar and controller.

  - Check that the correct COM port and baud rate are selected.