| Code | Setting | Valid range | Example |
| --- | --- | --- | --- |
| R | Local speed limit (display color threshold) | 0 .. 120 | @R_040# → set limit = 40 |
| K | Distance threshold (radar config) | 10 .. 100 | @K_050# → set distance = 50 |
| L | Direction filter (0/1/2) | 0 .. 2 | @L_1# → set direction = 1 00: Only detect away; 01: Only detect approach; 02: All detected |
| M | Speed threshold (radar config) | 0 .. 120 | @M_070# → set speed threshold = 70 |
| N | Detection delay (seconds) | 0 .. 30 | @N_05# → set delay = 5 |
| O | Trigger count | 0 .. 10 | @O_03# → set trigger count = 3 |
| P | Signal-to-Noise (SNR) threshold | 0 .. 255 | @P_010# → set SNR = 10 |

HLK-LD2451 — Client Quick Reference (serial setup)

1) Commands you can send (ASCII format)

Format: @<CODE>_<NUMBER>#
(Example: @M_050# or @R_40#)

Typical Responses on Terminal — What you will see and what it means

| Response seen on terminal | Meaning for user |
| --- | --- |
| Setting saved successfully! | The radar accepted your command and the new value is applied. |
| Setting NOT saved, retrying... | The radar did not accept the value. The controller will resend automatically. Try again later. |
| Failed to save setting after retries. | The controller tried multiple times and the radar did not accept the setting. |
| Invalid frame received! | The radar reply was corrupted or unexpected. Try resending the command. |
| ConfigFrame_SENT : <hex bytes ...> | Informational — shows the exact bytes sent to the radar (no action required). |
| Config_ACK : <hex bytes ...> or Config_END_ACK : <hex bytes ...> | Informational — radar replied while entering/exiting configuration mode. |
| Configuration_SENT : <hex bytes ...> or Configuration_Respone : <hex bytes ...> | Informational — shows config command/response bytes. |
| CONFIG_MODE_ENABLE | The radar entered configuration mode (ready to accept settings). |
| CONFIG_MODE_DISABLE | Configuration mode ended successfully. |
| BYTE SAVED SUCSESSFULLY | Local EEPROM on controller saved the value. |
| EEPROM_ERROR | Controller failed to save to local EEPROM (no radar change). |
| CMD: <your command> | Echo of the command you typed (confirms it was received by the controller). |
| data not received - LEDs turned off (timeout) | No radar data received for several seconds — display turned off. Check radar power/wiring. |
| No response / Timeout (implied by no message) | No reply arrived — check connection, power, COM port and baud rate. |

Quick Debug Checklist — step-by-step (for technicians / installers)

- Type the command

  - Enter the command exactly: @<CODE>_<NUMBER># and send (press Enter).

- Watch terminal for reply

  - Setting saved successfully! → Done.

  - Setting NOT saved, retrying... or Invalid frame received! → Re-send command once or twice.

  - No reply → move to step 3.

- If no reply or timeout

  - Confirm power to the radar and controller.

  - Check physical wiring (TX↔RX, RX↔TX) and common ground.

  - Ensure terminal is connected to the correct COM port and set to 115200 baud.

  - Restart power to radar and controller and try again.

- If setting keeps failing

  - Verify the number you entered is inside the valid range (see command table).

  - Try a different value to confirm behavior (e.g., set a small, known valid value).

  - If EEPROM messages appear (BYTE SAVED SUCSESSFULLY) but radar settings don’t change, recheck wiring and power then retry.

- Confirm operational data

  - After a successful setting, confirm the radar is producing target data (normal terminal output or live display).

  - If the display shows data not received - LEDs turned off (timeout) → verify radar data link and power.