| Command String | Parameter Name | Example Value | Description |
| --- | --- | --- | --- |
| @R_50# | Speed Limit | 50 | Sets the maximum speed limit (e.g., vehicles exceeding 50 km/h will be flagged). |
| @H_10# | Speed Threshold | 10 | Minimum speed threshold; ignores objects moving slower than 10 km/h. |
| @A_15# | Installation Angle | 15 | Radar mounting/installation angle in degrees. |
| @D_30# | Detection Distance | 30 | Maximum detection range in meters. |
| @N_01# | Detection Direction | 1 | Sets detection direction: 1 = approaching vehicles, 0 = receding vehicles, 2 = both. |
| @F_10# | Sampling Frequency | 10 | Radar sampling frequency (Hz). |
| @U_01# | Speed Unit | 1 | Sets speed display unit: 0 = km/h, 1 = mph. |
| @I_02# | Anti-Interference | 2 | Anti-interference level: adjusts radar sensitivity to external noise. |
| @P_01# | Speed Format | 1 | Output format of speed data: e.g., 0 = raw, 1 = formatted output. |

Typical Responses on Terminal

When you send a setting command, the radar gives a short response.
Here’s how to read it:

| Response Seen on Terminal | Meaning for User |
| --- | --- |
| Setting saved successfully! | The radar accepted your command and updated the setting. |
| Setting NOT saved, retrying... | The radar received the command but did not apply the value. Try again. |
| Invalid frame received! | The reply from radar was corrupted or incomplete. Try again. |
| No response / Timeout | The radar did not reply at all (check wiring, baud rate, or restart). |

Quick Debug Checklist

- Send Command: Type the string (e.g., @R_50#) in the terminal.

- Check Reply:

  - If you see “Setting saved successfully!” → Done.

  - If you see “NOT saved” or “Invalid frame” → resend the command.

  - If you see no response at all → check connection/cables.

- Re-Confirm Setting: You can send the same command again or request current config to verify.